## Reading climate control diagnostic trouble codes

The Automatic Climate Control which appeared in several models includes an easy-to-use 61-channel OBD (On Board Diagnostic) display which can be accessed without the use of special tools. This function can be an indispensable tool in diagnosing problems with the climate control system. Owners of cars without automatic climate control must have error codes read with a VAG 1551 Scan Tool. 

To Access CC OBD readout:

- Car ignition on

- Press and hold the recirculation button

- Press and hold the up arrow button

- Release both buttons: "01c" will be displayed ("01c" indicates channel 1, "02c" indicates channel 2, etc.)

To select a different channel, press temperature + or temperature - buttons. 

To call up information from a particular channel, select desired channel, and press the recirculation button. 

To leave memory display, press the AUTO button, or switch the ignition off.

### Diagnostic Channels and their descriptions

| Channel | Function |
| --- | --- |
| 1 | System malfunction - displayed as a Diagnostic Trouble Code (DTC) |
| 2 | Digital value of Interior Temperature Sensor in Headliner (G86) |
| 3 | Digital value of Interior Temperature Sensor in Instrument Panel (G56) |
| 4 | Digital value of Fresh Air Intake Duct Temperature Sensor (G89) |
| 5 | Digital value of Outside Air (Ambient) Temperature Sensor, front (G17) |
| 6 | Digital value of Outside Air (Ambient) Temperature Sensor |
| 7 | Digital value of Ambient Temperature Sensor at Fresh Air Blower (G109) |
| 8 | Digital value of Temperature Regulator Flap Motor Position Sensor (G92) |
| 9 | Delta value of Temperature Regulator Flap |
| 10 | Non-corrected specified value of Temperature Regulator Flap |
| 11 | Digital value of Central Flap Motor Position Sensor (G112) |
| 12 | Specified value for Central Flap |
| 13 | Digital value of Footwell/Defroster Flap Motor Position Sensor (G114) |
| 14 | Specified value of Footwell/Defroster Flap |
| 15 | Digital value of Back Pressure Flap Motor Position Sensor (G113) (Not installed in US/Canada models) |
| 16 | Specified value of Back Pressure Flap |
| 17 | Vehicle Speed (km/h) |
| 18 | Actual Fresh Air Blower voltage (Volts) |
| 19 | Specified Fresh Air Blower voltage (Volts) |
| 20 | A/C Compressor (A/C Clutch) voltage (Volts) |
| 21 | Number of low voltage occurrences, non-transient |
| 22 | Cycle condition of A/C Refrigerant High Pressure Switch (F118) |
| 23 | Cyclings of the A/C Refrigerant High Pressure Switch (F118) |
| 24 | Cyclings of the switches, absolute non-fluctuating |
| 25 | Analog/Digital value, Kick-Down Switch |
| 26 | Analog/Digital value, Engine Coolant Temperature (ECT) Warning Light |
| 27 | Coding value |
| 28 | Engine Speed (RPM) |
| 29 | A/C Compressor speed in RPM (Should equal Engine Speed x 1.28) |
| 30 | Software version |
| 31 | Display check (all segments of A/C Control Head display light up) |
| 32 | Position Sensor malfunction counter, Temperature Regulator Flap |
| 33 | Position Sensor malfunction counter, Central Flap |
| 34 | Position Sensor malfunction counter, Footwell/Defroster Flap |
| 35 | Position Sensor malfunction counter, Air Flow Flap |
| 36 | Feedback value, cold end-stop, Temperature Regulator Flap Motor Position Sensor (G92) |
| 37 | Feedback value, hot end-stop (maximum stop), Temperature Regulator Flap Motor Position Sensor (G92) |
| 38 | Feedback value, cold end-stop, Central Flap Motor Position Sensor (G112) |
| 39 | Feedback value, hot end-stop, Central Flap Motor Position Sensor (G112) |
| 40 | Feedback value, cold end-stop, Footwell/Defroster Flap Motor Position Sensor (G114) |
| 41 | Feedback value, hot end-stop, Footwell/Defroster Flap Motor Position Sensor (G114) |
| 42 | Feedback value, cold end-stop, Back Pressure Flap Motor Position Sensor (G113) (Not installed in US/Canada models) |
| 43 | Feedback value, hot end-stop, Back Pressure Flap Motor Position Sensor (G113) (Not installed in US/Canada models) |
| 44 | Vehicle operation cycle counter |
| 45 | Calculated interior temperature (internal software, in digits) |
| 46 | Outside (ambient) temperature, filtered, for regulation (internal software) |
| 47 | Outside (ambient) temperature, unfiltered, (internal software, in °C) |
| 48 | Outside (ambient) temperature, unfiltered (in digits) |
| 49 | Malfunction counter for speedometer (vehicle speed) signal |
| 50 | Standing time (in minutes) |
| 51 | Engine Coolant Temperature (ECT), in °C |
| 52 | Graphics channel 1, see chart below. NOTE: When Diagnostic Channel 52 is selected, .--.-. is displayed first. A/C compressor switch-off conditions are then identified by illuminated segments of the .88.8. display. |
| 53 | Graphics channel 2, see chart below. NOTE: When Diagnostic Channel 53 is selected, .--.-. is displayed first. A/C system electrical outputs are then identified by illuminated segments of the .88.8. display. |
| 54 | Control characteristics |
| 55 | Outside (ambient) temperature, in Celsius or Fahrenheit depending on the setting of the A/C control head |
| 56 | Temperature in Celsius, from Interior Temperature Sensor in Headliner (G86) |
| 57 | Temperature in Celsius, from Interior Temperature Sensor in Instrument Panel (G56) |
| 58 | Temperature in Celsius, from Fresh Air Intake Duct Temperature Sensor (G89) |
| 59 | Temperature in Celsius, from Outside Air (Ambient) Temperature Sensor, front (G17) |
| 60 | Temperature in Celsius, from Ambient Temperature Sensor at Fresh Air Blower (G109) |
| 61 | Software version (latest) |

### Diagnostic Trouble Codes

| DTC | Malfunction | Remarks |
| --- | --- | --- |
| 00.0 | No Malfunction Present |  |
| 02.1 | Interior Temperature Sensor in Headliner (G86), static open | Digital default value of 128 is used if sensor fails |
| 02.2 | Interior Temperature Sensor in Headliner (G86), static short |  |
| 02.3 | Interior Temperature Sensor in Headliner (G86), sporadic open |  |
| 02.4 | Interior Temperature Sensor in Headliner (G86), sporadic short |  |
| 03.1 | Interior Temperature Sensor in Instrument Panel (G56), static open | Digital default value of 128 is used if sensor fails |
| 03.2 | Interior Temperature Sensor in Instrument Panel (G56), static short |  |
| 03.3 | Interior Temperature Sensor in Instrument Panel (G56), sporadic open |  |
| 03.4 | Interior Temperature Sensor, in Instrument Panel (G56), sporadic short |  |
| 04.1 | Fresh Air Intake Duct Temperature Sensor (G89), static open | Value supplied by outside air temperature sensor (G17) is used if sensor fails |
| 04.2 | Fresh Air Intake Duct Temperature Sensor (G89), static short |  |
| 04.3 | Fresh Air Intake Duct Temperature Sensor (G89), sporadic open |  |
| 04.4 | Fresh Air Intake Duct Temperature Sensor (G89), sporadic short |  |
| 05.1 | Outside Air (Ambient) Temperature Sensor (G17), front, static open | Value supplied by fresh air intake duct temperature sensor (G89) is used if sensor fails Digital default value of 128 is used if sensors (G89) and (G17) both fail |
| 05.2 | Outside Air (Ambient) Temperature Sensor (G17), front, static short |  |
| 05.3 | Outside Air (Ambient) Temperature Sensor (G17), front, sporadic open |  |
| 05.4 | Outside Air (Ambient) Temperature Sensor (G17), front, sporadic short |  |
| 06.1 | Engine Coolant Temperature (ECT) (G110), static open | Engine coolant temperature is calculated if sensor should fail or is not installed; diagnosis occurs only above 0°C (32°F) |
| 06.2 | Engine Coolant Temperature (ECT) (G110), static short |  |
| 06.3 | Engine Coolant Temperature (ECT) (G110), sporadic open |  |
| 06.4 | Engine Coolant Temperature (ECT) (G110), sporadic short |  |
| 07.1 | Ambient Temperature Sensor at Fresh Air Blower (G109), static open | Programmed corrective value = 0 |
| 07.2 | Ambient Temperature Sensor at Fresh Air Blower (G109), static short |  |
| 07.3 | Ambient Temperature Sensor at Fresh Air Blower (G109), sporadic open |  |
| 07.4 | Ambient Temperature Sensor at Fresh Air Blower (G109), sporadic short |  |
| 08.1 | Temperature Regulator Flap Motor Position Sensor (G92), static open | 08.1 - 08.4: Temperature regulator flap motor will no longer be controlled automatically; manual adjustment only. |
| 08.2 | Temperature Regulator Flap Motor Position Sensor (G92), static short |  |
| 08.3 | Temperature Regulator Flap Motor Position Sensor (G92), sporadic open |  |
| 08.4 | Temperature Regulator Flap Motor Position Sensor (G92), sporadic short |  |
| 08.5 | Temperature Regulator Flap, static block | 08.5 - 08.7: Motor is cycled; software attempts to eliminate block. Watch Diagnostic Channel 8 to see digital value for flap motor position sensor (G92). |
| 08.6 | Temperature Regulator Flap Motor Position Sensor (G92), malfunction |  |
| 08.7 | Temperature Regulator Flap, sporadic block |  |
| 11.1 | Central Flap Motor Position Sensor (G112), static open | 11.1 - 11.4 Central flap motor will no longer be controlled automatically; manual adjustment only. |
| 11.2 | Central Flap Motor Position Sensor (G112), static short |  |
| 11.3 | Central Flap Motor Position Sensor (G112), sporadic open |  |
| 11.4 | Central Flap Motor Position Sensor (G112), sporadic short |  |
| 11.5 | Central Flap, static block | 11.5 - 11.7 Motor is cycled; software attempts to eliminate block. Watch Diagnostic Channel 11 to see digital value for central flap motor position sensor (G112). |
| 11.6 | Central Flap Motor Position Sensor (G112), malfunction |  |
| 11.7 | Central Flap, sporadic block |  |
| 13.1 | Footwell/Defroster Flap Motor Position Sensor (G114), static open | 13.1 - 13.4 Footwell/Defroster flap motor will no longer be controlled automatically; manual adjustment only. |
| 13.2 | Footwell/Defroster Flap Motor Position Sensor (G114), static short |  |
| 13.3 | Footwell/Defroster Flap Motor Position Sensor (G114), sporadic open |  |
| 13.4 | Footwell/Defroster Flap Motor Position Sensor (G114), sporadic short |  |
| 13.5 | Footwell/Defroster Flap, static block | 13.5 - 13.7 Motor is cycled; software attempts to eliminate block. Watch Diagnostic Channel 13 to see digital value for footwell/defroster flap position sensor (G114). |
| 13.6 | Footwell/Defroster Flap Motor Position Sensor (G114), malfunction |  |
| 13.7 | Footwell/Defroster Flap, sporadic block |  |
| 15.1 | Back Pressure Flap Motor Position sensor (G113), static open | 15.1 - 15.4 Digital value internally programmed for limp-home mode |
| 15.2 | Back Pressure Flap Motor Position Sensor (G113), static short |  |
| 15.3 | Back Pressure Flap Motor Position Sensor (G113), sporadic open |  |
| 15.4 | Back Pressure Flap Motor Position Sensor (G113), sporadic short |  |
| 15.5 | Back Pressure Flap, static block | 15.5 - 15.7 Motor is cycled; software attempts to eliminate block. Watch Diagnostic Channel 15 to see digital value for back pressure flap position sensor (G113). |
| 15.6 | Back Pressure Motor Position Sensor (G113), malfunction |  |
| 15.7 | Back Pressure Flap, sporadic block |  |
| 17.0 | Vehicle Speed Signal faulty |  |
| 18.1 | Fresh Air Blower voltage, static |  |
| 18.3 | Fresh Air Blower voltage, sporadic |  |
| 20.1 | A/C compressor voltage not OK - static | Compressor remains off until voltage is greater than 10.8V for at least 25 seconds |
| 20.3 | A/C compressor voltage not OK - sporadic |  |
| 22.1 | A/C Refrigerant High Pressure Switch (F118), static open | Compressor remains off until switch closes |
| 22.3 | A/C Refrigerant High Pressure Switch (F118), sporadic open |  |
| 22.5 | A/C Refrigerant High Pressure Switch (F118), 120X open | Compressor re-engagement circuit, VAG1551 Scan Tool function |
| 29.1 | Belt slip detection "soft," static |  |
| 29.2 | Belt slip detection "hard," static |  |
| 29.3 | Belt slip detection "soft," sporadic |  |
| 29.4 | Belt slip detection "hard," sporadic |  |

Diagnostic Channel 52 - Graphics Channel 1

A/C Compressor switch-off conditions 
Note: if an A/C compressor switch-off condition exists, a segment of the .88.8. display, noted below as an alpha-numeric code, will illuminate.

| Segment | Condition |
| --- | --- |
| A1 | Slippage or blockage, A/C Refrigerant High Pressure Switch, 120x off |
| B1 | Engine Speed (RPM) less than 200 - 500 |
| C1 | Engine Speed (RPM) greater than 6000 |
| D1 | Engine Speed (RPM) greater than 6000 |
| E1 | System function OK* |
| F1 | System function OK* |
| G1 | System function OK* |
| A2 | A/C manually switched off (A/C standby canceled) |
| B2 | Low Voltage |
| C2 | Kick-down switch (via Transmission Control Module (TCM), compressor off for 12 seconds max. |
| E2 | A/C Refrigerant Low Pressure Switch (F73) |
| F2 | A/C Refrigerant High Pressure Switch (F118) |
| G2 | System function OK* |
| A3 | ECON mode selected |
| B3 | OFF selected |
| C3 | Outside (ambient) temperature too low |
| D3 | Engine management system (compressor will remain off for 3 -12 seconds) |
| E3 | High pressure occurrences more than 30 times |
| F3 | Ambient Temperature Sensor at Fresh Air Blower (G109) less than 27°F (-3°C) |
| G3 | System function OK* |
| Decimal Point | Visible: A/C compressor ON Not Visible: A/C compressor OFF |

*segments g1, g2, and g3 must illuminate simultaneously to indicate system function is OK.

Diagnostic Channel 53 - Graphics Channel 2

Active electrical outputs 
Note: When a NO system electrical output is activated, a segment of the .88.8. display, noted below as an alpha-numeric code, will illuminate. The decimal point in the .88.8. display will not illuminate in this channel.

| Segment | Condition |
| --- | --- |
| A1 | Fan for interior temperature sensor |
| B1 | Fresh air/recirculation flap closed (recirculation mode) |
| C1 | Heater valve closed |
| D1 | Bi-directional wiring harness |
| E1 | A/C compressor ON |
| F1 | coolant fan first speed ON |
| G1 | System function OK* |
| A2 | Air flow flap open |
| B2 | Air flow flap open |
| C2 | Air flow flap closed |
| D2 | Footwell/Defroster flap in footwell position |
| E2 | Footwell/Defroster flap in footwell position |
| F2 | Footwell/Defroster flap in defroster position |
| G2 | System function OK* |
| A3 | Central flap in instrument panel outlet position |
| B3 | Central flap in instrument panel outlet position |
| C3 | Central flap in footwell outlet/defrost position |
| D3 | Temperature flap in cold air position |
| E3 | Temperature flap in cold air position |
| F3 | Temperature flap in warm air position |
| G3 | System function OK* |

* Segments g1, g2, and g3 must illuminate simultaneously to indicate system function is OK